/ ip address 

-----add address=192.168.4.110/24 network=192.168.4.0 broadcast=192.168.4.255 interface=ether1

-----add address=92.244.112.6/26 network=92.244.112.0 broadcast=92.244.112.63 interface=ether2 

add address=192.168.3.1/24 interface=ether3 

/ ip firewall mangle 

add chain=prerouting src-address-list=odd in-interface=ether1 action=mark-connection 

  new-connection-mark=odd passthrough=yes 

add chain=prerouting src-address-list=odd in-interface=ether1 action=mark-routing 

  new-routing-mark=odd 

/ ip firewall mangle 

add chain=prerouting src-address-list=even in-interface=ether1 action=mark-connection 

  new-connection-mark=even passthrough=yes 

add chain=prerouting src-address-list=even in-interface=ether1 action=mark-routing 

  new-routing-mark=even 

/ ip firewall mangle 

add chain=prerouting in-interface=ether1 connection-state=new nth=1,1,0 

    action=mark-connection new-connection-mark=odd passthrough=yes

add chain=prerouting in-interface=ether1 action=add-src-to-address-list

  address-list=odd address-list-timeout=1d connection-mark=odd passthrough=yes

add chain=prerouting in-interface=ether1 connection-mark=odd action=mark-routing 

    new-routing-mark=odd passthrough=no

/ ip firewall mangle 

add chain=prerouting in-interface=ether1 connection-state=new nth=1,1,1

    action=mark-connection new-connection-mark=even passthrough=yes

add chain=prerouting in-interface=ether1 action=add-src-to-address-list

  address-list=even address-list-timeout=1d connection-mark=even passthrough=yes 

add chain=prerouting in-interface=ether1 connection-mark=even action=mark-routing 

    new-routing-mark=even passthrough=no

add chain=prerouting in-interface=ether1 connection-state=new nth=1,1,1 

    src-address-list=!odd action=mark-connection new-connection-mark=even

    passthrough=yes

/ ip firewall nat 

add chain=srcnat connection-mark=odd action=src-nat to-addresses=92.244.112.6

    to-ports=0-65535 

add chain=srcnat connection-mark=even action=src-nat to-addresses=192.168.3.1

    to-ports=0-65535

/ ip route 

add dst-address=0.0.0.0/0 gateway=92.244.112.1 scope=255 target-scope=10 routing-mark=odd 

add dst-address=0.0.0.0/0 gateway=192.168.3.110 scope=255 target-scope=10 routing-mark=even

/ ip route

add dst-address=0.0.0.0/0 gateway=92.244.112.1 scope=255 target-scope=10

